برنامه ای به زبان متلب بنویسید که یک خرپای دو بعدی با اغضای دلخواه را از ورودی گرفته و تحت بارگذاری مشخص تغییر مکانها نقاط گرهی و همچنین تنش ها و کرنشهای اعضا را محاسبه کند.

مثال مورد بررسی

برنامه به گونه ای نوشته شده است که با تغییر اطلاعان ورودی شامل مختصات گرهی، اتصالات و مشخصات اعضا ( سطح مقطع و مدول الاستیسته ) می تواند برای هر نوع خرپای دو بعدی استفاده شود. کدها در پیوست ارائه شده است.

نتایج:

- خروجی متلب:

Dislacement at DOF 1 = -0.033023

Dislacement at DOF 2 = -0.12407

Dislacement at DOF 3 = -0.012033

Dislacement at DOF 4 = -0.032675

Dislacement at DOF 5 = 0.00086957

Dislacement at DOF 6 = -0.012033

Dislacement at DOF 7 = 0

Dislacement at DOF 8 = 0

Dislacement at DOF 9 = 0.032458

Dislacement at DOF 10 = 0.012297

Dislacement at DOF 11 = 0.012468

Dislacement at DOF 12 = 0.018617

Dislacement at DOF 13 = 0

Dislacement at DOF 14 = 0

Element 1 Force  :  8000

Element 1 Stress  :  40000000

Element 1 Strain  :  0.00017391

Element 2 Force  :  0

Element 2 Stress  :  0

Element 2 Strain  :  0

Element 3 Force  :  -20000

Element 3 Stress  :  -100000000

Element 3 Strain  :  -0.00043478

Element 4 Force  :  -8944.2719

Element 4 Stress  :  -44721359.55

Element 4 Strain  :  -0.00019444

Element 5 Force  :  -26832.8157

Element 5 Stress  :  -134164078.65

Element 5 Strain  :  -0.00058332

Element 6 Force  :  -58137.7674

Element 6 Stress  :  -290688837.075

Element 6 Strain  :  -0.0012639

Element 7 Force  :  -10000

Element 7 Stress  :  -50000000

Element 7 Strain  :  -0.00021739

Element 8 Force  :  10000

Element 8 Stress  :  50000000

Element 8 Strain  :  0.00021739

Element 9 Force  :  -22360.6798

Element 9 Stress  :  -111803398.875

Element 9 Strain  :  -0.0004861

Element 10 Force  :  -70710.6781

Element 10 Stress  :  -353553390.5933

Element 10 Strain  :  -0.0015372

- شکل تغییر شکل یافته

پیوست کدهای متلب

clc;

clear;

%% Input

% Module of elasticity

E=2.3e11  ;

% Nodes  [ label   x   y]

Nodes = [1,0,3;

2,2,3;

3,4,3;

4,6,3;

5,2,2;

6,4,1;

7,6,0];

%Elements  [ elementlabel nodei nodej A]

Elements = [1 1 2 200e-6;

2 2 3 200e-6;

3 3 4 200e-6;

4 1 5 200e-6;

5 5 6 200e-6;

6 6 7 200e-6;

7 2 5 200e-6;

8 3 6 200e-6;

9 3 5 200e-6;

10 4 6 200e-6;];

% Loading [ node   fx   fy]

L = [1 , 0 , -4000;

2 , 0 , -2000];

% Support [ node   dx   dy] ( 1 --> restrained , 0 ---> free)

S = [4 , 1 , 1;

7 , 1 , 1];

%% Do the calculation

num_Nodes = size(Nodes);num_Nodes = num_Nodes(1,1);

num_Elements = size(Elements);num_Elements = num_Elements(1,1);

num_DOF = 2;

num_ALLDOF = num_Nodes*num_DOF;

% Force Matrix

num_loads = size(L);num_loads = num_loads(1,1);

F = zeros(num_ALLDOF,1);

for i = 1:num_loads

nodenumber = L(i,1);

% Node degrees of freedom : [ nodeNumber*2-1 , nodeNumber*2] for example

% for Node 2 degrees of freedom become [ 3,4]

dof_1 = nodenumber*2-1;

dof_2 = nodenumber*2;

F(dof_1) = L(i,2);

F(dof_2) = L(i,3);

end

% Find Restrained DOFs

num_R = size(S);num_R = num_R(1,1);

DOF_R = [];

for i = 1:num_R

nodenumber = S(i,1);

% Node degrees of freedom : [ nodeNumber*2-1 , nodeNumber*2] for example

% for Node 2 degrees of freedom become [ 3,4]

dof_1 = nodenumber*2-1;

if S(i,2) == 1 % if dof_1 restrained

DOF_R = [DOF_R;dof_1];

end

dof_2 = nodenumber*2;

if S(i,3) == 1 % if dof_2 restrained

DOF_R = [DOF_R;dof_2];

end

end

% Find Free DOFs  = DOF_F

DOF_All = (1:num_ALLDOF)';

DOF_F = DOF_All;

DOF_F(DOF_R) = [];

% Construct Structure Stiffnes Matrix

K = fcn_GlobalStiff_2D(Elements,Nodes,E);

% Remove constraint degrees of freedom from stiffeness matrix

K_F = K;

K_F(DOF_R,:) =[];

K_F(:,DOF_R) =[];

% Free Force matrix ( Loda at free degrees of freedom)

F_F=F;

F_F(DOF_R) = [];

% calculate displacements

D_F=K_F\F_F;

% Displacement at all degrees of freedom

Dtotal=zeros(num_ALLDOF,1);

Dtotal(DOF_F) = D_F;

% Display Node Displacements at free DOFs

for i = 1:length(Dtotal)

disp(['Dislacement at DOF ' ,num2str(i) , ' = ', num2str(Dtotal(i))]);

end

% calculate elements internal forces

for ii=1:num_Elements

nodei=Elements(ii,2); % element node label at end i

nodej=Elements(ii,3); % element node label at end j

% Extract element ends coordinates

xi=Nodes(nodei,2);

yi=Nodes(nodei,3);

xj=Nodes(nodej,2);

yj=Nodes(nodej,3);

% calculate element length

L=sqrt((xj-xi)^2+(yj-yi)^2);

% Construct element local stiffness matrix

A=Elements(ii,4);

K_element = fcn_LocalStiff_2D(A,L,E);

% element degrees of freedom

dos=[nodei*2-1,nodei*2,nodej*2-1,nodej*2];

% extrcact dof displacements assossiate with element end nodes

D_element=zeros(4,1);

for jj=1:4

D_element(jj)=Dtotal(dos(jj));

end

% element transform matrix

T = fcn_TransMstrix_2D(xi,yi,xj,yj);

% transform  displacements to element local coordinates

D_element_local=T*D_element;

% calculate element internal forces

F_element_local=K_element*D_element_local;

F_element_global=T'*F_element_local;

Ele_Force = F_element_local(4);

disp([' Element ', num2str(ii),' Force  ' , ':  ' , num2str(Ele_Force)]);

disp([' Element ', num2str(ii),' Stress  ' ,':  ' , num2str(Ele_Force/A)]);

disp([' Element ', num2str(ii),' Strain  ' ,':  ' , num2str(Ele_Force/A/E)]);

end

% Plot

scale = 2;

for ii=1:num_Elements

nodei=Elements(ii,2); % element node label at end i

nodej=Elements(ii,3); % element node label at end j

% Extract element ends coordinates

xi=Nodes(nodei,2);

yi=Nodes(nodei,3);

xj=Nodes(nodej,2);

yj=Nodes(nodej,3);

hold on;

plot([xi,xj],[yi,yj],'-ok','LineWidth',2);

dos=[nodei*2-1,nodei*2,nodej*2-1,nodej*2];

dxi = Dtotal(dos(1))*scale;

dyi = Dtotal(dos(2))*scale;

dxj = Dtotal(dos(3))*scale;

dyj = Dtotal(dos(4))*scale;

xi2 = xi+dxi;

yi2 = yi + dyi;

xj2 = xj+dxj;

yj2 = yj+dyj;

plot([xi2,xj2],[yi2,yj2],'-b','LineWidth',1);

end

axis equal

a = xlim;

b = ylim;

xmin = a(1) - 0.1*(a(2) - a(1));

xmax = a(2) + 0.1*(a(2) - a(1));

ymin = b(1) - 0.1*(b(2) - b(1));

ymax = b(2) + 0.1*(b(2) - b(1));

xlim([xmin,xmax]);

ylim([ymin,ymax]);

grid on;