برنامه ای به زبان متلب بنویسید که یک خرپای دو بعدی با اغضای دلخواه را از ورودی گرفته و تحت بارگذاری مشخص تغییر مکانها نقاط گرهی و همچنین تنش ها و کرنشهای اعضا را محاسبه کند.

مثال مورد بررسی

برنامه به گونه ای نوشته شده است که با تغییر اطلاعان ورودی شامل مختصات گرهی، اتصالات و مشخصات اعضا ( سطح مقطع و مدول الاستیسته ) می تواند برای هر نوع خرپای دو بعدی استفاده شود. کدها در پیوست ارائه شده است.

نتایج:

- خروجی متلب:

_________________________________

Dislacements:

_________________________________

Dislacement at Node 1:

Dx = 0

Dy = 0

Dislacement at Node 2:

Dx = 0.00075458

Dy = -0.034489

Dislacement at Node 3:

Dx = -5.2769e-18

Dy = -0.04253

Dislacement at Node 4:

Dx = -0.00075458

Dy = -0.034489

Dislacement at Node 5:

Dx = 0

Dy = 0

Dislacement at Node 6:

Dx = -0.0088083

Dy = -0.029011

Dislacement at Node 7:

Dx = -1.0731e-18

Dy = -0.039512

Dislacement at Node 8:

Dx = 0.0088083

Dy = -0.029011

_________________________________

Reactions:

_________________________________

Reaction at Node 1:

Rx = -99507.7216

Ry = 90000

Reaction at Node 5:

Rx = 99507.7216

Ry = 90000

_________________________________

Element Stress and Strain:

_________________________________

Element 1 Stress  :  15846202.6599

Element 1 Strain  :  7.5458e-05

Element 2 Stress  :  -15846202.6599

Element 2 Strain  :  -7.5458e-05

Element 3 Stress  :  -15846202.6599

Element 3 Strain  :  -7.5458e-05

Element 4 Stress  :  15846202.6599

Element 4 Strain  :  7.5458e-05

Element 5 Stress  :  212132034.356

Element 5 Strain  :  0.0010102

Element 6 Stress  :  -115025738.6532

Element 6 Strain  :  -0.00054774

Element 7 Stress  :  44819829.4276

Element 7 Strain  :  0.00021343

Element 8 Stress  :  -49461074.7306

Element 8 Strain  :  -0.00023553

Element 9 Stress  :  -63384810.6397

Element 9 Strain  :  -0.00030183

Element 10 Stress  :  -49461074.7306

Element 10 Strain  :  -0.00023553

Element 11 Stress  :  44819829.4276

Element 11 Strain  :  0.00021343

Element 12 Stress  :  -115025738.6532

Element 12 Strain  :  -0.00054774

Element 13 Stress  :  212132034.356

Element 13 Strain  :  0.0010102

Element 14 Stress  :  184974261.3468

Element 14 Strain  :  0.00088083

Element 15 Stress  :  184974261.3468

Element 15 Strain  :  0.00088083

- شکل تغییر شکل یافته

پیوست کدهای متلب

clc;

clear;

%% 1 - Input

% Module of elasticity

E=2.1e11 ;

node_filename = 'Nodes.txt';

element_filename = 'Elements.txt';

load_filename = 'Loading.txt';

rest_filename = 'Restraints.txt';

% Import Data

Nodes = load(node_filename);

Elements= load(element_filename);

Loads = load(load_filename);

Rests = load(rest_filename);

%% 2- Create Free and Fix Degrees of Freedom vectors

% Number of Nodes

NN = size(Nodes);

NN = NN(1,1);

% All Degrees of Freedom

DOF_Truss = 1:NN*2;

DOF_Free = DOF_Truss;

DOF_Rest = [];

% Number of fixed nodes

NR = size(Rests);

NR = NR(1,1);

for i = 1: NR

n = Rests(i,1) ; % Node label

f1 = Rests(i,2) ;

f2 = Rests(i,3) ;

dofx = n*2-1; % degree of freedom in x direction

dofy = n*2; % degree of freedom in y direction

if f1 == 1 % if node is fixed in x direction

DOF_Rest = [DOF_Rest,dofx];

end

if f2 == 1 % if node is fixed in y direction

DOF_Rest = [DOF_Rest,dofy];

end

end

% Remode fixed dof from all dofs to create free dof vector

DOF_Free(DOF_Rest) = [];

%% 3- Force vector and free force vector

NL = size(Loads);

NL = NL(1,1);

F = zeros(NN*2,1);

for i =1:NL

n = Loads(i,1) ; % Node label

fx = Loads(i,2) ;

fy = Loads(i,3) ;

dofx = n*2-1; % degree of freedom in x direction

dofy = n*2; % degree of freedom in y direction

F(dofx) = fx;

F(dofy) = fy;

end

% Remove fixed dof from force vector

FF = F;

FF(DOF_Rest) = [];

%% 4- Truss Stiffeness matrix

NE = size(Elements);

NE = NE(1,1);

K = zeros(2*NN,2*NN);

for i = 1: NE

node_i = Elements(i,1);

node_j = Elements(i,2);

A = Elements(i,3);

xi = Nodes(node_i,1);

yi = Nodes(node_i,2);

xj = Nodes(node_j,1);

yj = Nodes(node_j,2);

L = sqrt((xj-xi)^2 +(yj-yi)^2);

cx = (xj-xi)/L;

cy = (yj-yi)/L;

dof1 = node_i*2-1;

dof2 = node_i*2;

dof3 = node_j*2-1;

dof4 = node_j*2;

K(dof1,dof1) = K(dof1,dof1)  + (A*E/L)*(cx^2);

K(dof1,dof2) = K(dof1,dof2) + (A*E/L)*(cx*cy);

K(dof1,dof3) = K(dof1,dof3) - (A*E/L)*(cx^2);

K(dof1,dof4) = K(dof1,dof4) - (A*E/L)*(cx*cy);

K(dof2,dof1) = K(dof2,dof1) + (A*E/L)*(cx*cy);

K(dof2,dof2) = K(dof2,dof2) + (A*E/L)*(cy^2);

K(dof2,dof3) = K(dof2,dof3) - (A*E/L)*(cx*cy);

K(dof2,dof4) = K(dof2,dof4) - (A*E/L)*(cy^2);

K(dof3,dof1) = K(dof3,dof1) - (A*E/L)*(cx^2);

K(dof3,dof2) = K(dof3,dof2) - (A*E/L)*(cx*cy);

K(dof3,dof3) = K(dof3,dof3) + (A*E/L)*(cx^2);

K(dof3,dof4) = K(dof3,dof4) + (A*E/L)*(cx*cy);

K(dof4,dof1) = K(dof4,dof1) - (A*E/L)*(cx*cy);

K(dof4,dof2) = K(dof4,dof2) - (A*E/L)*(cy^2);

K(dof4,dof3) = K(dof4,dof3) + (A*E/L)*(cx*cy);

K(dof4,dof4) = K(dof4,dof4) + (A*E/L)*(cy^2);

end

% Remove fixed degrees of freedom from stiffeness matrix

KF = K;

KF(DOF_Rest,:) =[];

KF(:,DOF_Rest) =[];

%% 5- Calculate Displacements at free degrees of freedom and create displacement vector

DF=KF\FF;

D=zeros(NN*2,1);

D(DOF_Free) = DF;

disp('_________________________________');

disp('Dislacements: ');

disp('_________________________________');

for i=1:NN

dofx = i*2-1; % degree of freedom in x direction

dofy = i*2; % degree of freedom in y direction

disp(['Dislacement at Node ' ,num2str(i) , ': ']);

disp(['Dx = ' ,num2str(D(dofx)) ]);

disp(['Dy = ' ,num2str(D(dofy)) ]);

end

%% 6- calculate Reactions

R = K*D-F;

disp('_________________________________');

disp('Reactions: ');

disp('_________________________________');

for i=1:NR

n = Rests(i,1);

dofx = n*2-1; % degree of freedom in x direction

dofy = n*2; % degree of freedom in y direction

disp(['Reaction at Node ' ,num2str(n) , ': ']);

disp(['Rx = ' ,num2str(R(dofx))]);

disp(['Ry = ' ,num2str(R(dofy))]);

end

%% 6- calculate elements Forces, Stresses and Straines

disp('_________________________________');

disp('Element Stress and Strain: ');

disp('_________________________________');

for i=1:NE

node_i = Elements(i,1);

node_j = Elements(i,2);

A = Elements(i,3);

xi = Nodes(node_i,1);

yi = Nodes(node_i,2);

xj = Nodes(node_j,1);

yj = Nodes(node_j,2);

L = sqrt((xj-xi)^2 +(yj-yi)^2);

cx = (xj-xi)/L;

cy = (yj-yi)/L;

dof1 = node_i*2-1;

dof2 = node_i*2;

dof3 = node_j*2-1;

dof4 = node_j*2;

D_e = D([dof1;dof2;dof3;dof4]);

Stress = (E/L) * [-cx,-cy,cx,cy]*D_e;

disp([' Element ', num2str(i),' Stress  ' ,':  ' , num2str(Stress)]);

disp([' Element ', num2str(i),' Strain  ' ,':  ' , num2str(Stress/E)]);

end

% Plot

scale = 20;

for i=1:NE

node_i = Elements(i,1);

node_j = Elements(i,2);

xi = Nodes(node_i,1);

yi = Nodes(node_i,2);

xj = Nodes(node_j,1);

yj = Nodes(node_j,2);

hold on;

plot([xi,xj],[yi,yj],'--ob','LineWidth',2);

dof1 = node_i*2-1;

dof2 = node_i*2;

dof3 = node_j*2-1;

dof4 = node_j*2;

dxi = D(dof1)*scale;

dyi = D(dof2)*scale;

dxj = D(dof3)*scale;

dyj = D(dof4)*scale;

xi2 = xi+dxi;

yi2 = yi + dyi;

xj2 = xj+dxj;

yj2 = yj+dyj;

plot([xi2,xj2],[yi2,yj2],'-or','LineWidth',2);

end

axis equal

a = xlim;

b = ylim;

xmin = a(1) - 0.1*(a(2) - a(1));

xmax = a(2) + 0.1*(a(2) - a(1));

ymin = b(1) - 0.1*(b(2) - b(1));

ymax = b(2) + 0.1*(b(2) - b(1));

xlim([xmin,xmax]);

ylim([ymin,ymax]);

grid on;