برنامه ای به زبان متلب بنویسید که یک خرپای دو بعدی با اغضای دلخواه را از ورودی گرفته و تحت بارگذاری مشخص تغییر مکانها نقاط گرهی و همچنین تنش ها و کرنشهای اعضا را محاسبه کند.

مثال مورد بررسی

برنامه به گونه ای نوشته شده است که با تغییر اطلاعان ورودی شامل مختصات گرهی، اتصالات و مشخصات اعضا ( سطح مقطع و مدول الاستیسته ) می تواند برای هر نوع خرپای دو بعدی استفاده شود. کدها در پیوست ارائه شده است.

نتایج:

- خروجی متلب:

Dislacements:

Node          Disp X          Disp y

----          ------           ------

1          0                  0

2          1.0672e-18                  -0.009523

3          1.0326e-18                  -0.011428

4          8.5429e-19                  -0.009523

5          0                  0

6          0.0020336                  -0.0080945

7          1.9057e-19                  -0.009523

8          -0.0020336                  -0.0080945

Stress/Strain:

Element          Stress           Strain

----             ------           ------

1          7.4704e-08                  3.5574e-19

2          -2.4223e-09                  -1.1535e-20

3          -1.2482e-08                  -5.9438e-20

4          -5.98e-08                  -2.8476e-19

5          -212132034.356                  -0.0010102

6          100000000                  0.00047619

7          1.7881e-07                  8.5149e-19

8          100000000                  0.00047619

9          -5.9605e-08                  -2.8383e-19

10          100000000                  0.00047619

11          -212132034.356                  -0.0010102

12          -158113883.0084                  -0.00075292

13          -158113883.0084                  -0.00075292

- شکل تغییر شکل یافته

پیوست کدهای متلب

clc;

clear;

% Read Data from escel file ( Truss.xlsx)

Nodes = readmatrix('Truss.xlsx','Sheet','Nodes');

Elements= readmatrix('Truss.xlsx','Sheet','Connectivity');

Loads = readmatrix('Truss.xlsx','Sheet','Loads');

Support = readmatrix('Truss.xlsx','Sheet','Support');

% direction cosines table , Element Degrees of freedom of End Nodes

n_ele = size(Elements); n_ele=n_ele(1,1);

DC_table = zeros(n_ele,3);

Ele_DOF_table = zeros(n_ele,4);

for i = 1: n_ele

ni = Elements(i,2);

nj = Elements(i,3);

xi = Nodes(ni,2);

yi = Nodes(ni,3);

xj = Nodes(nj,2);

yj = Nodes(nj,3);

le = sqrt((xj-xi)^2 +(yj-yi)^2);

l = (xj-xi)/le;

m = (yj-yi)/le;

DC_table(i,:) = [le, l , m];

Ele_DOF_table(i,:) = [ni*2-1, ni*2 ,nj*2-1, nj*2];

end

% Construct Stiffeness matrix of truss

n_node = size(Nodes);n_node=n_node(1,1);

K = zeros(n_node*2,n_node*2);

for i = 1: n_ele

A = Elements(i,4);

E = Elements(i,5);

ke = K_Global(E,A,DC_table(i,1),DC_table(i,2),DC_table(i,3));

dofs = Ele_DOF_table(i,:);

for j = 1: 4

for k =1:4

K(dofs(j),dofs(k)) = K(dofs(j),dofs(k)) + ke(j,k);

end

end

end

%

% Define dof correspond to fixed support

DOF_fixed = [];

DOF_free = [];

n_sup = size(Support);n_sup=n_sup(1,1);

for i = 1:n_node

node = Support(i,1);

if Support(i,2) == 1

DOF_fixed =[DOF_fixed,node*2-1];

else

DOF_free =[DOF_free,node*2-1];

end

if Support(i,3) == 1

DOF_fixed =[DOF_fixed,node*2];

else

DOF_free =[DOF_free,node*2];

end

end

% elimination columns and rows of stiffeness matrix correspond to fixed support

K_Free = K;

K_Free(DOF_fixed ,:) = [];

K_Free( : ,DOF_fixed) = [];

% Construct Force Matrix

F = zeros(n_node*2,1);

for i = 1: n_node

node = Loads(i,1);

F(node*2-1) = Loads(i,2);

F(node*2) = Loads(i,3);

end

% elimination Forces correspond to fixed support

F_Free = F;

F_Free(DOF_fixed ,:) = [];

% Calculate Displacements corresponds to free degrees of freedom

Q_Free=K_Free\F_Free;

% nodal displacement vector for the entire structure

Q_All = zeros(n_node*2,1);

Q_All(DOF_free) = Q_Free;

% Calculate Stresses and Strains

Stress = zeros(n_ele,1);

Stains = zeros(n_ele,1);

for i = 1: n_ele

E = Elements(i,5);

le = DC_table(i,1);

l = DC_table(i,2);

m = DC_table(i,3);

dofs = Ele_DOF_table(i,:);

q = Q_All(dofs);

Stress(i) = (E/le)*[-l,-m,l,m]*q;

Stains(i) = Stress(i)./E;

end

% Support Reactions

R = K*Q_All - F;

R_supports = R(DOF_fixed);

% Display Results

disp('Dislacements: ');

disp('Node          Disp X          Disp y ');

disp('----          ------           ------');

for i=1:n_node

dof_1 = i*2-1;

dof_2 = i*2;

disp([num2str(i) ,'          ', num2str(Q_All(dof_1)),'                  ', num2str(Q_All(dof_2))]);

end

disp('Stress/Strain: ');

disp('Element          Stress           Strain ');

disp('----             ------           ------');

for i=1:n_ele

disp([num2str(i) ,'          ', num2str(Stress(i)),'                  ', num2str(Stains(i))]);

end

% Plot

scale = 100;

for i=1:n_ele

ni = Elements(i,2);

nj = Elements(i,3);

xi = Nodes(ni,2);

yi = Nodes(ni,3);

xj = Nodes(nj,2);

yj = Nodes(nj,3);

hold on;

plot([xi,xj],[yi,yj],'-ob','LineWidth',2);

dofs = Ele_DOF_table(i,:);

dxi = Q_All(dofs(1))*scale;

dyi = Q_All(dofs(2))*scale;

dxj = Q_All(dofs(3))*scale;

dyj = Q_All(dofs(4))*scale;

xi2 = xi+dxi;

yi2 = yi + dyi;

xj2 = xj+dxj;

yj2 = yj+dyj;

plot([xi2,xj2],[yi2,yj2],'-r','LineWidth',1);

end

axis equal

a = xlim;

b = ylim;

xmin = a(1) - 0.1*(a(2) - a(1));

xmax = a(2) + 0.1*(a(2) - a(1));

ymin = b(1) - 0.1*(b(2) - b(1));

ymax = b(2) + 0.1*(b(2) - b(1));

xlim([xmin,xmax]);

ylim([ymin,ymax]);

grid on;