برنامه ای به زبان متلب بنویسید که یک خرپای دو بعدی با اغضای دلخواه را از ورودی گرفته و تحت بارگذاری مشخص تغییر مکانها نقاط گرهی و همچنین تنش ها و کرنشهای اعضا را محاسبه کند.

مثال مورد بررسی

برنامه به گونه ای نوشته شده است که با تغییر اطلاعان ورودی شامل مختصات گرهی، اتصالات و مشخصات اعضا ( سطح مقطع و مدول الاستیسته ) می تواند برای هر نوع خرپای دو بعدی استفاده شود. کدها در پیوست ارائه شده است.

نتایج:

- خروجی متلب:

Node 1 Displacement:  -0.0074286  0

Node 1 Reaction:  3.7494e-11  15000

Node 2 Displacement:  -0.006  -0.01443

Node 2 Reaction:  -1.2699e-11  -7.5639e-11

Node 3 Displacement:  -0.0045714  -0.018355

Node 3 Reaction:  -2.4859e-11  -4.2529e-11

Node 4 Displacement:  -0.0037143  -0.01781

Node 4 Reaction:  -2.8069e-11  -5.1173e-11

Node 5 Displacement:  -0.0028571  -0.018355

Node 5 Reaction:  1.8137e-11  -1.219e-10

Node 6 Displacement:  -0.0014286  -0.01443

Node 6 Reaction:  -1.0349e-12  -7.722e-11

Node 7 Displacement:  0  0

Node 7 Reaction:  -3.5846e-12  15000

Node 8 Displacement:  -0.0022099  -0.01443

Node 8 Reaction:  -7.1371e-11  8.0945e-11

Node 9 Displacement:  -0.0017019  -0.018641

Node 9 Reaction:  2.5851e-11  8.5493e-11

Node 10 Displacement:  -0.0037143  -0.01781

Node 10 Reaction:  -3.4164e-11  3.1832e-11

Node 11 Displacement:  -0.0057266  -0.018641

Node 11 Reaction:  5.4623e-11  8.1855e-11

Node 12 Displacement:  -0.0052187  -0.01443

Node 12 Reaction:  6.1792e-11  1.5461e-11

Element 1 Stress:  150000000

Element 1 Strain:  0.00071429

Element 2 Stress:  150000000

Element 2 Strain:  0.00071429

Element 3 Stress:  90000000

Element 3 Strain:  0.00042857

Element 4 Stress:  90000000

Element 4 Strain:  0.00042857

Element 5 Stress:  150000000

Element 5 Strain:  0.00071429

Element 6 Stress:  150000000

Element 6 Strain:  0.00071429

Element 7 Stress:  -167705098.3125

Element 7 Strain:  -0.0007986

Element 8 Stress:  4.7684e-07

Element 8 Strain:  2.2707e-18

Element 9 Stress:  -33541019.6625

Element 9 Strain:  -0.00015972

Element 10 Stress:  -30000000

Element 10 Strain:  -0.00014286

Element 11 Stress:  54083269.132

Element 11 Strain:  0.00025754

Element 12 Stress:  2.3842e-07

Element 12 Strain:  1.1353e-18

Element 13 Stress:  54083269.132

Element 13 Strain:  0.00025754

Element 14 Stress:  -30000000

Element 14 Strain:  -0.00014286

Element 15 Stress:  -33541019.6625

Element 15 Strain:  -0.00015972

Element 16 Stress:  0

Element 16 Strain:  0

Element 17 Stress:  -167705098.3125

Element 17 Strain:  -0.0007986

Element 18 Stress:  -134164078.65

Element 18 Strain:  -0.00063888

Element 19 Stress:  -134164078.65

Element 19 Strain:  -0.00063888

Element 20 Stress:  -134164078.65

Element 20 Strain:  -0.00063888

Element 21 Stress:  -134164078.65

Element 21 Strain:  -0.00063888

- شکل تغییر شکل یافته

پیوست کدهای متلب

%%

clc;

clear;

%% Input

E = 2.1E11;

Node_Data = readmatrix('StructureData.xlsx','Sheet',1);

Element_Data= readmatrix('StructureData.xlsx','Sheet',2);

%% Collect requard matrix and vectors

Ele_Numbers = size(Element_Data);

Ele_Numbers=Ele_Numbers(1,1);

Node_number = size(Node_Data);

Node_number=Node_number(1,1);

% 1- Nodal degrees of freedom

Node_DOFs = zeros(Node_number,3);

Free_DOF =[];

Fixed_DOF =[];

count_free = 1;

count_fix = 1;

for i = 1: Node_number

Node_DOFs(i,1) = i;

dof_x = 2*i-1; % dof at x

dof_y = 2*i; % % dof at y

Node_DOFs(i,2) = dof_x; % dof at x

Node_DOFs(i,3) = dof_y; % dof at y

if Node_Data (i,6) == 1

Fixed_DOF(count_fix) = dof_x;

count_fix = count_fix+1;

else

Free_DOF(count_free) = dof_x;

count_free = count_free+1;

end

if Node_Data (i,7) == 1

Fixed_DOF(count_fix) = dof_y;

count_fix = count_fix+1;

else

Free_DOF(count_free) = dof_y;

count_free = count_free+1;

end

end

% 2 - Nodal Forces

F_All = zeros(Node_number*2,1);

for i = 1: Node_number

dof_x = 2*i-1; % dof at x

dof_y = 2*i; % % dof at y

F_All(dof_x) = Node_Data(i,4);

F_All(dof_y) = Node_Data(i,5);

end

F_FreeDOF = F_All(Free_DOF);

% 3- Element Data

Ele_Data = zeros(Ele_Numbers,8);

for i = 1 : Ele_Numbers

node_i = Element_Data(i,2);

node_j = Element_Data(i,3);

A = Element_Data(i,4);

xi = Node_Data(node_i,2);

yi = Node_Data(node_i,3);

xj = Node_Data(node_j,2);

yj = Node_Data(node_j,3);

L = sqrt((xj-xi)^2 +(yj-yi)^2);

C = (xj-xi)/L;

S = (yj-yi)/L;

Ele_Data(i,:) = [i,A,L,C,S,C^2,S^2,C*S];

end

%% Do Calculation

% 1- Structure Stiffeness Matrix

K_All = zeros(Node_number*2,Node_number*2);

for i = 1 : Ele_Numbers

% Element Stiffeness matris

A = Ele_Data(i,2);

L = Ele_Data(i,3);

c2 = Ele_Data(i,6);

s2 = Ele_Data(i,7);

cs = Ele_Data(i,8);

k_e = K_Element(E,A,L,c2,s2,cs);

node_i = Element_Data(i,2);

node_j = Element_Data(i,3);

dof_e(1) = Node_DOFs(node_i,2);

dof_e(2) = Node_DOFs(node_i,3);

dof_e(3) = Node_DOFs(node_j,2);

dof_e(4) = Node_DOFs(node_j,3);

for j = 1:4

for k = 1:4

K_All(dof_e(j),dof_e(k)) =  K_All(dof_e(j),dof_e(k)) + k_e(j,k);

end

end

end

% 2- Eliminate Fixed degrees of freedom from Structure Stiffeness Matrix

K_FreeDof = K_All;

K_FreeDof(Fixed_DOF,:) = [];

K_FreeDof(:,Fixed_DOF) = [];

% 3- Calculate Displacements at Free Degrees of freedom

Disp_FreeDOF = K_FreeDof\F_FreeDOF;

% 4 - Calculate Displacements at all Degrees of freedom

Disp_All =  zeros(Node_number*2,1);

Disp_All(Free_DOF) = Disp_FreeDOF;

% 5- calculate Reaction Forces

R_All = K_All*Disp_All-F_All;

% 6- Calculate element Stress and Strains

Ele_Stress =zeros(Ele_Numbers,1);

Ele_Strain =zeros(Ele_Numbers,1);

for i = 1 : Ele_Numbers

L = Ele_Data(i,3);

c = Ele_Data(i,4);

s = Ele_Data(i,5);

Cp = (E/L).*[-c -s c s];

node_i = Element_Data(i,2);

node_j = Element_Data(i,3);

dof_e(1) = Node_DOFs(node_i,2);

dof_e(2) = Node_DOFs(node_i,3);

dof_e(3) = Node_DOFs(node_j,2);

dof_e(4) = Node_DOFs(node_j,3);

d = Disp_All(dof_e);

Ele_Stress(i) = Cp*d;

Ele_Strain(i) = Ele_Stress(i)/E;

end

% 7- Display results

% Nodal Displacements and reactions

for i = 1:Node_number

node = Node_DOFs(i,1);

dof_x = Node_DOFs(i,2);

dof_y = Node_DOFs(i,3);

disp(['Node ' , num2str(node),' Displacement:  ' ,num2str(Disp_All(dof_x)),'  ',num2str(Disp_All(dof_y))]);

disp(['Node ' , num2str(node),' Reaction:  ' ,num2str(R_All(dof_x)),'  ',num2str(R_All(dof_y))]);

end

% Element Sress and Strain

for i = 1:Ele_Numbers

disp(['Element ' , num2str(i),' Stress:  ' ,num2str(Ele_Stress(i))]);

disp(['Element ' , num2str(i),' Strain:  ' ,num2str(Ele_Strain(i))]);

end

% Plot

scale = 50;

for i=1:Ele_Numbers

node_i = Element_Data(i,2);

node_j = Element_Data(i,3);

xi = Node_Data(node_i,2);

yi = Node_Data(node_i,3);

xj = Node_Data(node_j,2);

yj = Node_Data(node_j,3);

hold on;

plot([xi,xj],[yi,yj],'-ob','LineWidth',2);

dof_e(1) = Node_DOFs(node_i,2);

dof_e(2) = Node_DOFs(node_i,3);

dof_e(3) = Node_DOFs(node_j,2);

dof_e(4) = Node_DOFs(node_j,3);

dxi = Disp_All(dof_e(1))*scale;

dyi = Disp_All(dof_e(2))*scale;

dxj = Disp_All(dof_e(3))*scale;

dyj = Disp_All(dof_e(4))*scale;

xi2 = xi+dxi;

yi2 = yi + dyi;

xj2 = xj+dxj;

yj2 = yj+dyj;

plot([xi2,xj2],[yi2,yj2],'-r','LineWidth',1);

end

axis equal

a = xlim;

b = ylim;

xmin = a(1) - 0.1*(a(2) - a(1));

xmax = a(2) + 0.1*(a(2) - a(1));

ymin = b(1) - 0.1*(b(2) - b(1));

ymax = b(2) + 0.1*(b(2) - b(1));

xlim([xmin,xmax]);

ylim([ymin,ymax]);

grid on;